Lesson 15 – Finite State Machines (FSM) - Design

From Last Time:

- What do Flip-Flops Create?  Memory!

- When do they do this?  On Clock Edge (rising vs falling)

Finite State Machine (FSM) Makeup:  M- inputs, N – Outputs, and K - bits of state

- FSMs have K registers that can be one of a finite number (2K) unique states

Two Types of FSMs:

Moore Machine – outputs depend only on current state of the machine.

Mealy Machine – outputs depend on both the current state and current inputs of the machine.

Real Life Examples of State Machines:

- Soda Machine

- Software Applications

- Spell Checker – Using Soundex

- Driving to School/Work

How to Design a State Machine:

- Description

- State Transition Diagram

- State Transition Table & Output Table

- Next State and Output State Equations

- Schematic

FSM Example – Vending Machine:

Step 0)  Description

- Only takes Quarters.  Only sells (insert favorite drink).

- No change.  50₵ per Soda

- Inputs:  C = Coin (also, there’s a clock)

- Outputs:  S = Soda

Step 1)  State Transition Diagram (Moore Machine)

- Transitions only occur with rising edge of clock.

- We have 3-4 states How many F/Fs do we need?

2 Lets name them A (MSB) and B (LSB) (i.e. 22)

***Need to account for all states

- The ghost state could be our reset state

- When we turn on machine we need to account for all possible states

| State Table: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Current State |  |  | Next State |  |
| A | B | C (Coin) | A* | B* |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 |

Step 2)  State Transition Table & Output Table

| Output Table |  |  |
| --- | --- | --- |
| Current State |  | Output |
| A | B | S |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Step 3)  Next State and Output State Equations

S=AB

A*=ABC

B*=AB+AC

Step 4)  Schematic

Step 1)  State Transition Diagram (Mealy Machine)

- Potential for less states

- More difficult to design

- Outputs tied to arcs

How many Flip-Flops do we need now? 1

Step 2)  State Transition Table & Output Table

| State/Output Table: |  |  |  |
| --- | --- | --- | --- |
| Current State |  | Next State | Output |
| State | C (Coin) | State* | Tea |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Step 3)  Next State and Output State Equations

Tea=SC

S*=SC+SC

Step 4)  Schematic

Boardwork:

- Draw me a picture of a Mealy or Moore State Transition Diagram that shows

- A Snail is crawling across a sequence of numbers and he smiles if the last 2 numbers are “11”

- Sequence:  0100110111

Example 3.5: Synchronous Sequential Circuits

Solutions

Circuit:

- (a) is combinational, not sequential, because it has no registers.

- (b) is a simple sequential circuit with no feedback.

- (c) is neither a combinational circuit nor a synchronous sequential circuit, because it has a latch that is neither a register nor a combinational circuit.

- (d) and (e) are synchronous sequential logic; they are two forms of finite state machines, which are discussed in Section 3.4.

- Same as (d)

- (f) is neither combinational nor synchronous sequential, because it has a cyclic path from the output of the combinational logic back to the input of the same logic but no register in the path.

- (g) is synchronous sequential logic in the form of a pipeline, which we will study in Section 3.6.

- (h) is not, strictly speaking, a synchronous sequential circuit, because the second register receives a different clock signal than the first, delayed by two inverter delays.