Lesson 16 – Finite State Machines (FSM) - Analysis

Analysis

Design

State Machine:

- Description

- State Transition Diagram

- State Transition Table & Output Table

- Next State and Output State Equations

- Schematic

Slide Questions:

1)  True it is a Moore Machine  How would a Mealy look Different?

2)  Moore Machine

3)  D1*=Q2+Q1

4)  Next State=S2  Output (Y)=0

5)  True  Look at State Machine Design/Analysis Table

6)  3 Flip-Flops (i.e. 23 = 8)

7)  8 states (5 Normal States and 3 Ghost States)

8)	a)  No -  No register

b)  No – One cyclic path doesn’t have a register

c)  Yes -

9)  C – K-maps

10)  B – Determine the next-state and output functions implemented by the circuit

Finite State Machine (FSM) Analysis – Joe Homeowner’s Garage door Opener:

Step 4)  Schematic

| Next State Table: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Current State |  | Inputs |  | Next State |  | Next State Name |
| S1 | S0 | A1 | A0 | S1* | S0* |  |
| 0 | 0 | 0 | 0 | 0 | 0 | M0 |
| 0 | 0 | 0 | 1 | 0 | 0 | M0 |
| 0 | 0 | 1 | 0 | 0 | 0 | M0 |
| 0 | 0 | 1 | 1 | 0 | 1 | M1 |
| 0 | 1 | 0 | 0 | 0 | 0 | M0 |
| 0 | 1 | 0 | 1 | 1 | 0 | M2 |
| 0 | 1 | 1 | 0 | 0 | 0 | M0 |
| 0 | 1 | 1 | 1 | 0 | 0 | M0 |
| 1 | 0 | 0 | 0 | 0 | 0 | M0 |
| 1 | 0 | 0 | 1 | 0 | 0 | M0 |
| 1 | 0 | 1 | 0 | 0 | 0 | M0 |
| 1 | 0 | 1 | 1 | 0 | 0 | M0 |
| 1 | 1 | 0 | 0 | 0 | 0 | N/A |
| 1 | 1 | 0 | 1 | 1 | 0 | N/A |
| 1 | 1 | 1 | 0 | 0 | 0 | N/A |
| 1 | 1 | 1 | 1 | 0 | 0 | N/A |

Step 3)  Next State and Output State Equations

Next State Equations

S1*=S0A1A0

S0*=S1S0A1A0

Output Equation:

M0

Unlock=S1

Step 2)  State Transition Table & Output Table

| Output Table |  |  |  |
| --- | --- | --- | --- |
| Current State |  | Output | State Name |
| S1 | S0 | Unlock |  |
| 0 | 0 | 0 | M0 |
| 0 | 1 | 0 | M1 |
| 1 | 0 | 1 | M2 |
| 1 | 1 | 1 | M3 |

M1

M2

M3 Ghost State – never gets here

Step 1)  State Transition Diagram

31

Step 0)  Description

What is the Door combination?

Step 4)  Schematic

| Next State Table: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Current State |  | Inputs | Next State |  | Next State |
| S1 | S0 | X | S1* | S0* | Name |
| 0 | 0 | 0 | 1 | 0 | M2 |
| 0 | 0 | 1 | 1 | 1 | M3 |
| 0 | 1 | 0 | 1 | 0 | M2 |
| 0 | 1 | 1 | 1 | 0 | M2 |
| 1 | 0 | 0 | 0 | 0 | M0 |
| 1 | 0 | 1 | 0 | 1 | M1 |
| 1 | 1 | 0 | 1 | 0 | M2 |
| 1 | 1 | 1 | 1 | 0 | M2 |

Step 3)  Next State and Output State Equations

Next State Equations

S1*=S1+S0

S0*=XS0

M0

Output Equation:

Q=S1+S0

M1

Step 2)  State Transition Table & Output Table

| Output Table |  |  |  |
| --- | --- | --- | --- |
| Current State |  | Output | State Name |
| S1 | S0 | Q |  |
| 0 | 0 | 0 | M0 |
| 0 | 1 | 1 | M1 |
| 1 | 0 | 1 | M2 |
| 1 | 1 | 1 | M3 |

M2

M3

Step 1)  State Transition Diagram

Step 0)  Description

This finite state machine is a divide-by-two counter (see Section 3.4.2) when X = 0. When X = 1, the output, Q, is HIGH.

when X = 0. When X = 1, the output, Q, is HIGH.

Example 3.6:

Divide-by-N counter has one output and no inputs. The output Y is HIGH for one clock cycle out of every N. In other words, the output divides the frequency of the clock by N.

The waveform and state transition diagram for a divide-by-3 counter is shown in Figure 3.28. Sketch circuit designs for such a counter using binary and one-hot state encodings.

| Next State Table |  |
| --- | --- |
| Current State | Next State |
| S0 | S1 |
| S1 | S2 |
| S2 | S0 |

| Output Table |  |
| --- | --- |
| Current State | Output |
| S0 | 1 |
| S1 | 0 |
| S2 | 0 |