What is the Difference between Sequential and Combinational Logic Circuits?

- Sequential has memory (i.e. Flip-Flops)

Counter:

Flip-Flop – adds memory so we remember where we are where we have been

Shift Register:

- Creates a serial-to-parallel conversion

- Shifts into LSB and outputs 4-Bits (N-Bits) given a 4-Bit System

What is the difference between a Shifter and a Shift Register?

- A shifter is purely combinational logic

- Address – Indicates which row of memory is read/written                           (location) 2N (N-Bits)

- Data – Value read/written M-Bits

- Word – size of row of data (M-Bits)

- Depth – Number of rows

- Width – Number of Columns

- Wordline – Assert based on address to activate bit cells in a row

- Bitline – kept at Z; when wordline is activated, driven to 1 or 0 by bit cell to read

  - When written , drives bit cell high or low

- Port – Gives read/write access to the address; multi-ported can read/write several addresses at the same time

Memory

Address –

Data –

Word –

Depth -

Width –

Wordline –

Bitline –

ROM – Read Only Memory

    - Also randomly accessed

    - Many can be written now

    - Non-Volatile

RAM – Random Access Memory

    - Delay same for all addresses, vs sequential (e.g. type)

    - Volatile – Loses its data when the power is lost

Port –

Types of memory:

RAM –

ROM –

| Types of RAM: DRAM – DRAM – Dynamic RAM - Bit stored as absence or presence of charge on a capacitor - Reading destroys data; must be rewritten - Must be refreshed every few miliseconds due to leaking charge Pppppppp0 b m SRAM – SRAM – Static RAM - Does not need refreshing - Bit stored in cross coupled inverters - Inverters can restore value if noise interferes | Types of DRAM: SDRAM – Synchronous DRAM - Uses pipelined memory access DDR SDRAM – Double data rate SDRAM - Uses Rising and falling edge to access data SDRAM – DDR SDRAM – Memory Type; Transistors per bit cell; Latency. Flip-Flop; ~20; Fast. SRAM; 6; Medium. DRAM; 1; Slow. Memory Type; Transistors per bit cell; Latency. - Larger memories tend to be slower - More transistors = more size, power, and cost - Best Memory depends on speed, cost, power constraints |
| --- | --- |

Register file -

Register File – Group of registers that stores temporary variables

  - Often consists of small multiported SRAM

PROM

Fuse Programmable ROM

Types of ROM:

PROM – Programmable Read only Memory– Provides a way to connect transistor with ground

PROM –

Fuse Programmable ROM – blow fuse = 1,  cannot be rewritten

Fuse-programmable ROM –

EPROM – Eraseable PROM

  - High voltage to write

  - UV light to erase

EPROM –

EEPROM – Electrically EPROM

  - Bit cells individually erased

  - Erased without UV light with extra circuitry

EEPROM –

Flash – Large blocks erased

  - Cheaper because less erasing circuitry

Flash –

Look Up Table!

Logic using memory arrays: